// core_top_fpga.v - FPGA专用顶层（基于core_top_sim.v完整功能）

// Target FPGA: xc7k325tffg900-2

// Clock: 200 MHz differential (AD12/AD11) → converted to single-ended clk_i

// UART TX: Y23 (LVCMOS33)

`timescale 1ns/1ps

module core_top_fpga (

// 差分系统时钟输入（来自板载200MHz晶振）

input  wire        clk_p_i,     // SYSCLK_P @ AD12

input  wire        clk_n_i,     // SYSCLK_N @ AD11

// UART发送（FPGA → 外部）

output wire        uart_tx_o,

// SPI 接口（可连接到外部SPI Flash或传感器）

output wire        spi_sclk_o,

output wire        spi_mosi_o,

input  wire        spi_miso_i,

output wire        spi_cs_o,

// I2C 接口（可连接到外部I2C设备）

inout  wire        i2c_sda_io,

inout  wire        i2c_scl_io,

// GPIO 接口（32位，可根据需要引出部分引脚）

inout  wire [31:0] gpio_io,

// LED outputs for debugging（8个LED）

output wire        led0_o,  // T28

output wire        led1_o,  // V19

output wire        led2_o,  // U30

output wire        led3_o,  // U29

output wire        led4_o,  // V20

output wire        led5_o,  // V26

output wire        led6_o,  // W24

output wire        led7_o   // W23

);

// ==========================================================================

// Step 1: 将差分时钟转换为单端时钟（供CPU使用）

// ==========================================================================

wire clk_200m;  // 单端200MHz时钟，供给整个CPU

IBUFDS u_ibufds_clk (

.I(clk_p_i),          // 正端

.IB(clk_n_i),         // 负端

.O(clk_200m)          // 输出单端时钟

);

// ==========================================================================

// Step 2: 内部上电复位生成

// ==========================================================================

reg [24:0] rst_counter = 0;

wire rst_n_internal;

always @(posedge clk_200m) begin

if (!rst_n_internal) begin

rst_counter <= rst_counter + 1;

end

end

assign rst_n_internal = &rst_counter;  // 所有位为1时才解除复位

// ==========================================================================

// Step 3: 内部信号声明（从core_top_sim.v复制）

// ==========================================================================

// IFU

wire [31:0] if_pc;

wire [31:0] if_instr;

wire [31:0] pc_plus4;

// IF/ID

wire [31:0] if_id_pc;

wire [31:0] if_id_instr;

// ID

wire [31:0] id_rs1_data;

wire [31:0] id_rs2_data;

wire [31:0] id_imm;

wire [4:0]  id_rs1_addr;

wire [4:0]  id_rs2_addr;

wire [4:0]  id_rd_addr;

wire [3:0]  id_alu_op;

wire        id_alu_src;

wire        id_mem_we;

wire        id_mem_re;

wire [1:0]  id_wb_sel;

wire        id_reg_we;

wire        id_branch;

wire        id_jump;

wire [2:0]  id_funct3;

wire [2:0]  id_mem_width;

wire [6:0]  id_opcode;

wire        id_csr_inst;

wire [11:0] id_csr_addr;

wire [2:0]  id_csr_op;

wire [4:0]  id_csr_zimm;

// ID/EX

wire [31:0] id_ex_pc;

wire [31:0] id_ex_rs1_data;

wire [31:0] id_ex_rs2_data;

wire [31:0] id_ex_imm;

wire [4:0]  id_ex_rs1_addr;

wire [4:0]  id_ex_rs2_addr;

wire [4:0]  id_ex_rd_addr;

wire [3:0]  id_ex_alu_op;

wire        id_ex_alu_src;

wire        id_ex_mem_we;

wire        id_ex_mem_re;

wire [1:0]  id_ex_wb_sel;

wire        id_ex_reg_we;

wire        id_ex_branch;

wire        id_ex_jump;

wire [2:0]  id_ex_funct3;

wire [2:0]  id_ex_mem_width;

wire [6:0]  id_ex_opcode;

wire        id_ex_mret;

wire        id_ex_csr_inst;

wire [11:0] id_ex_csr_addr;

wire [2:0]  id_ex_csr_op;

wire [4:0]  id_ex_csr_zimm;

// EX

wire [31:0] ex_alu_result;

wire [31:0] ex_mem_addr;

wire [31:0] ex_mem_wdata;

wire        ex_mem_we;

wire        ex_mem_re;

wire [2:0]  ex_mem_width;

wire        ex_branch_taken;

wire        ex_jump_taken;

wire [31:0] ex_branch_target;

wire [31:0] ex_jump_target;

wire [31:0] ex_pc_plus4;

wire [31:0] ex_result;

wire [1:0]  ex_wb_sel;

wire        ex_reg_we;

wire [4:0]  ex_rd_addr;

wire [31:0] op1_selected;

wire [31:0] op2_selected;

wire [31:0] ex_csr_result;

wire [31:0] forward_mem_data;

// EX/MEM

wire [31:0] ex_mem_alu_result;

wire [31:0] ex_mem_mem_addr;

wire [31:0] ex_mem_mem_wdata;

wire        ex_mem_mem_we;

wire        ex_mem_mem_re;

wire [2:0]  ex_mem_mem_width;

wire [31:0] ex_mem_pc_plus4;

wire [4:0]  ex_mem_rd_addr;

wire [1:0]  ex_mem_wb_sel;

wire        ex_mem_reg_we;

wire [31:0] ex_mem_csr_result;

wire [4:0]  ex_mem_rd_addr_for_hazard;

wire        ex_mem_reg_we_for_hazard;

wire        ex_mem_mem_re_for_hazard;

// MEM

wire [31:0] mem_alu_result;

wire [31:0] mem_mem_rdata;

wire [31:0] mem_pc_plus4;

wire [4:0]  mem_rd_addr;

wire [1:0]  mem_wb_sel;

wire        mem_reg_we;

wire [31:0] mem_csr_result;

// MEM/WB

wire [31:0] wb_alu_result;

wire [31:0] wb_mem_rdata;

wire [31:0] wb_pc_plus4;

wire [4:0]  wb_rd_addr;

wire [1:0]  wb_wb_sel;

wire        wb_reg_we;

wire [31:0] wb_csr_result;

wire        mem_wb_mem_re;

// WB

wire [31:0] wb_data;

wire        wb_reg_we_out;

wire [4:0]  wb_rd_addr_out;

// MEM Bus Interface

wire        mem_bus_re;

wire        mem_bus_we;

wire [31:0] mem_bus_addr;

wire [31:0] mem_bus_wdata;

wire [2:0]  mem_bus_width;

wire [31:0] mem_bus_rdata;

wire        mem_bus_ready;

// Bus Signals

wire        bus_ram_re;

wire        bus_ram_we;

wire [31:0] bus_ram_addr;

wire [31:0] bus_ram_wdata;

wire [2:0]  bus_ram_width;

wire [31:0] bus_ram_rdata;

wire        bus_ram_ready;

wire        bus_uart_we;

wire        bus_uart_re;

wire [31:0] bus_uart_addr;

wire [31:0] bus_uart_wdata;

wire [31:0] bus_uart_rdata;

wire        bus_gpio_we;

wire        bus_gpio_re;

wire [31:0] bus_gpio_addr;

wire [31:0] bus_gpio_wdata;

wire [31:0] bus_gpio_rdata;

wire        bus_timer_we;

wire        bus_timer_re;

wire [31:0] bus_timer_addr;

wire [31:0] bus_timer_wdata;

wire [31:0] bus_timer_rdata;

wire        bus_spi_we;

wire        bus_spi_re;

wire [31:0] bus_spi_addr;

wire [31:0] bus_spi_wdata;

wire [31:0] bus_spi_rdata;

wire        bus_i2c_we;

wire        bus_i2c_re;

wire [31:0] bus_i2c_addr;

wire [31:0] bus_i2c_wdata;

wire [31:0] bus_i2c_rdata;

// UART internal signals

wire [7:0]  tx_data;

wire        tx_valid;

wire        tx_ready;

wire        uart_interrupt;

// SPI internal signals

wire        spi_interrupt;

wire [1:0]  spi_state;

wire [7:0]  spi_tx_data;

wire [7:0]  spi_rx_data;

// I2C internal signals

wire        i2c_interrupt;

wire [2:0]  i2c_state;

wire [7:0]  i2c_tx_data;

wire [7:0]  i2c_rx_data;

wire        i2c_ack;

// GPIO internal signals

wire        gpio_interrupt;

wire [31:0] gpio_out;

wire [31:0] gpio_oe;

wire [31:0] gpio_in;

// Timer internal signals

wire        timer_interrupt;

wire [31:0] timer_load_val;

wire [31:0] timer_count_val;

wire        timer_enable;

wire        timer_irq_flag;

// Hazard & Forwarding

wire [1:0] forwardA;

wire [1:0] forwardB;

wire       stall_if;

wire       stall_id;

wire       flush_if;

wire       flush_id;

wire       stall_ex;

wire       stall_mem;

wire       stall_wb;

wire       flush_ex;

wire       flush_mem;

wire       flush_wb;

wire        hazard_load_use;

wire        hazard_control;

wire [4:0]  hazard_rs1_addr;

wire [4:0]  hazard_rs2_addr;

wire [4:0]  hazard_ex_rd_addr;

wire        hazard_ex_reg_we;

wire        hazard_ex_mem_re;

wire [4:0]  fwd_ex_mem_rd;

wire        fwd_ex_mem_reg_we;

wire [4:0]  fwd_mem_wb_rd;

wire        fwd_mem_wb_reg_we;

// ==========================================================================

// 中断相关信号

// ==========================================================================

wire       intr_software = 1'b0;

wire       intr_timer = timer_interrupt;

wire       intr_external = gpio_interrupt || spi_interrupt || i2c_interrupt;

wire [31:0] csr_rdata;

wire [31:0] mtvec;

wire [31:0] mepc;

wire [31:0] mcause;

wire [31:0] mie;

wire [31:0] mstatus;

wire [31:0] mip;

wire        id_mret;

wire        csr_inst_valid;

wire [2:0]  csr_op;

wire [11:0] csr_addr;

wire [4:0]  csr_rs1_addr;

wire [31:0] csr_rs1_data;

wire [31:0] csr_imm;

wire [31:0] csr_inst_result;

wire        csr_inst_we;

wire [11:0] csr_inst_waddr;

wire [31:0] csr_inst_wdata;

wire        csr_we;

wire [11:0] csr_waddr;

wire [31:0] csr_wdata;

wire [31:0] csr_result;

wire        intr_pending;

wire [31:0] intr_cause;

wire [31:0] intr_handler_addr;

wire        pipe_csr_mepc_we;

wire [31:0] pipe_csr_mepc_data;

wire        pipe_csr_mcause_we;

wire [31:0] pipe_csr_mcause_data;

wire        pipe_csr_mstatus_we;

wire [31:0] pipe_csr_mstatus_data;

wire        interrupt_taken_pipe;

wire        interrupt_flush_pipe;

wire [31:0] interrupt_pc_pipe;

wire        intr_flush_if;

wire        intr_flush_id;

wire        intr_flush_ex;

wire        intr_flush_mem;

wire        intr_flush_wb;

// GPIO 三态缓冲控制

wire [31:0] gpio_out_reg;

wire [31:0] gpio_oe_reg;

wire [31:0] gpio_in_reg;

genvar g;

generate

for (g = 0; g < 32; g = g + 1) begin : gpio_tristate

IOBUF u_gpio_iobuf (

.I(gpio_out_reg[g]),

.O(gpio_in_reg[g]),

.IO(gpio_io[g]),

.T(~gpio_oe_reg[g])

);

end

endgenerate

// ==========================================================================

// Step 4: 实例化所有模块

// ==========================================================================

// Data RAM

data_ram u_data_ram (

.clk_i     (clk_200m),

.rst_n_i   (rst_n_internal),

.we_i      (bus_ram_we),

.re_i      (bus_ram_re),

.width_i   (bus_ram_width),

.addr_i    (bus_ram_addr),

.wdata_i   (bus_ram_wdata),

.rdata_o   (bus_ram_rdata),

.ready_o   (bus_ram_ready)

);

assign stall_ex  = 1'b0;

assign stall_mem = 1'b0;

assign stall_wb  = 1'b0;

assign flush_ex  = ex_branch_taken || ex_jump_taken;

assign flush_mem = 1'b0;

assign flush_wb  = 1'b0;

// IFU

ifu_top u_ifu_top (

.clk              (clk_200m),

.rst_n            (rst_n_internal),

.stall_i          (stall_if || intr_flush_if),

.branch_taken_i   (ex_branch_taken),

.jump_taken_i     (ex_jump_taken),

.branch_target_i  (ex_branch_target),

.jump_target_i    (ex_jump_target),

.interrupt_pending_i(interrupt_taken_pipe),

.mtvec_i          (mtvec),

.instr            (if_instr),

.pc               (if_pc),

.pc_plus4         (pc_plus4)

);

// IF/ID Register

if_id_reg u_if_id_reg (

.clk_i           (clk_200m),

.rst_n_i         (rst_n_internal),

.stall_i         (stall_id),

.flush_i         (flush_id),

.intr_flush_i    (intr_flush_id),

.if_pc_i         (if_pc),

.if_instr_i      (if_instr),

.id_pc_o         (if_id_pc),

.id_instr_o      (if_id_instr)

);

// ID Top

id_top u_id_top (

.clk           (clk_200m),

.rst_n         (rst_n_internal),

.instr         (if_id_instr),

.pc            (if_id_pc),

.wb_we_i       (wb_reg_we_out),

.wb_rd_addr_i  (wb_rd_addr_out),

.wb_rd_data_i  (wb_data),

.rs1_data_o    (id_rs1_data),

.rs2_data_o    (id_rs2_data),

.imm_o         (id_imm),

.rs1_addr_o    (id_rs1_addr),

.rs2_addr_o    (id_rs2_addr),

.rd_addr_o     (id_rd_addr),

.alu_op_o      (id_alu_op),

.alu_src_o     (id_alu_src),

.mem_we_o      (id_mem_we),

.mem_re_o      (id_mem_re),

.wb_sel_o      (id_wb_sel),

.reg_we_o      (id_reg_we),

.branch_o      (id_branch),

.jump_o        (id_jump),

.funct3_o      (id_funct3),

.opcode_o      (id_opcode),

.mem_width_o   (id_mem_width),

.mret_o        (id_mret),

.csr_inst_o    (id_csr_inst),

.csr_addr_o    (id_csr_addr),

.csr_op_o      (id_csr_op),

.csr_zimm_o    (id_csr_zimm)

);

// ID/EX Register

id_ex_reg u_id_ex_reg (

.clk_i           (clk_200m),

.rst_n_i         (rst_n_internal),

.stall_i         (stall_id),

.flush_i         (flush_id),

.intr_flush_i    (intr_flush_ex),

.id_pc_i         (if_id_pc),

.id_rs1_data_i   (id_rs1_data),

.id_rs2_data_i   (id_rs2_data),

.id_imm_i        (id_imm),

.id_rs1_addr_i   (id_rs1_addr),

.id_rs2_addr_i   (id_rs2_addr),

.id_rd_addr_i    (id_rd_addr),

.id_alu_op_i     (id_alu_op),

.id_alu_src_i    (id_alu_src),

.id_mem_we_i     (id_mem_we),

.id_mem_re_i     (id_mem_re),

.id_mem_width_i  (id_mem_width),

.id_wb_sel_i     (id_wb_sel),

.id_reg_we_i     (id_reg_we),

.id_branch_i     (id_branch),

.id_jump_i       (id_jump),

.id_funct3_i     (id_funct3),

.id_opcode_i     (id_opcode),

.id_csr_inst_i   (id_csr_inst),

.id_csr_addr_i   (id_csr_addr),

.id_csr_op_i     (id_csr_op),

.id_csr_zimm_i   (id_csr_zimm),

.id_mret_i       (id_mret),

.ex_mret_o       (id_ex_mret),

.ex_csr_inst_o   (id_ex_csr_inst),

.ex_csr_addr_o   (id_ex_csr_addr),

.ex_csr_op_o     (id_ex_csr_op),

.ex_csr_zimm_o   (id_ex_csr_zimm),

.ex_pc_o         (id_ex_pc),

.ex_rs1_data_o   (id_ex_rs1_data),

.ex_rs2_data_o   (id_ex_rs2_data),

.ex_imm_o        (id_ex_imm),

.ex_rs1_addr_o   (id_ex_rs1_addr),

.ex_rs2_addr_o   (id_ex_rs2_addr),

.ex_rd_addr_o    (id_ex_rd_addr),

.ex_alu_op_o     (id_ex_alu_op),

.ex_alu_src_o    (id_ex_alu_src),

.ex_mem_we_o     (id_ex_mem_we),

.ex_mem_re_o     (id_ex_mem_re),

.ex_mem_width_o  (id_ex_mem_width),

.ex_wb_sel_o     (id_ex_wb_sel),

.ex_reg_we_o     (id_ex_reg_we),

.ex_branch_o     (id_ex_branch),

.ex_jump_o       (id_ex_jump),

.ex_funct3_o     (id_ex_funct3),

.ex_opcode_o     (id_ex_opcode)

);

// Hazard Unit

hazard_unit u_hazard_unit (

.clk_i            (clk_200m),

.rst_n_i          (rst_n_internal),

.id_rs1_addr_i    (id_rs1_addr),

.id_rs2_addr_i    (id_rs2_addr),

.id_ex_rd_addr_i  (id_ex_rd_addr),

.id_ex_reg_we_i   (id_ex_reg_we),

.id_ex_mem_re_i   (id_ex_mem_re),

.ex_mem_rd_addr_i (ex_mem_rd_addr_for_hazard),

.ex_mem_reg_we_i  (ex_mem_reg_we_for_hazard),

.ex_mem_mem_re_i  (ex_mem_mem_re_for_hazard),

.branch_taken_i   (ex_branch_taken),

.jump_taken_i     (ex_jump_taken),

.interrupt_taken_i(interrupt_taken_pipe),

.interrupt_flush_i(interrupt_flush_pipe),

.stall_if_o       (stall_if),

.stall_id_o       (stall_id),

.flush_if_o       (flush_if),

.flush_id_o       (flush_id),

.intr_flush_if_o  (intr_flush_if),

.intr_flush_id_o  (intr_flush_id),

.intr_flush_ex_o  (intr_flush_ex),

.intr_flush_mem_o (intr_flush_mem),

.intr_flush_wb_o  (intr_flush_wb)

);

// Forwarding Unit

forwarding_unit u_forwarding_unit (

.id_ex_rs1_addr_i   (id_ex_rs1_addr),

.id_ex_rs2_addr_i   (id_ex_rs2_addr),

.ex_mem_rd_addr_i   (ex_mem_rd_addr),

.ex_mem_reg_we_i    (ex_mem_reg_we),

.ex_mem_mem_re_i    (ex_mem_mem_re),

.mem_wb_rd_addr_i   (wb_rd_addr),

.mem_wb_reg_we_i    (wb_reg_we),

.stall_i            (stall_id),

.forwardA_o         (forwardA),

.forwardB_o         (forwardB)

);

// EX Top

ex_top u_ex_top (

.clk_i              (clk_200m),

.rst_n_i            (rst_n_internal),

.rs1_data_i         (id_ex_rs1_data),

.rs2_data_i         (id_ex_rs2_data),

.imm_i              (id_ex_imm),

.pc_i               (id_ex_pc),

.wb_sel_i           (id_ex_wb_sel),

.reg_we_i           (id_ex_reg_we),

.rd_addr_i          (id_ex_rd_addr),

.alu_op_i           (id_ex_alu_op),

.alu_src_i          (id_ex_alu_src),

.branch_i           (id_ex_branch),

.jump_i             (id_ex_jump),

.funct3_i           (id_ex_funct3),

.mem_we_i           (id_ex_mem_we),

.mem_re_i           (id_ex_mem_re),

.mem_width_i        (id_ex_mem_width),

.ex_forward_data_i  (ex_mem_alu_result),

.mem_forward_data_i (forward_mem_data),

.forwardA_i         (forwardA),

.forwardB_i         (forwardB),

.opcode_i           (id_ex_opcode),

.csr_result_i       (csr_result),

.alu_result_o       (ex_alu_result),

.mem_addr_o         (ex_mem_addr),

.mem_wdata_o        (ex_mem_wdata),

.mem_we_o           (ex_mem_we),

.mem_re_o           (ex_mem_re),

.branch_taken_o     (ex_branch_taken),

.branch_target_o    (ex_branch_target),

.jump_taken_o       (ex_jump_taken),

.jump_target_o      (ex_jump_target),

.pc_plus4_o         (ex_pc_plus4),

.ex_result_o        (ex_result),

.wb_sel_o           (ex_wb_sel),

.reg_we_o           (ex_reg_we),

.rd_addr_o          (ex_rd_addr),

.mem_width_o        (ex_mem_width),

.op1_selected_o     (op1_selected),

.op2_selected_o     (op2_selected),

.ex_csr_result_o    (ex_csr_result),

.csr_mepc_i         (mepc),

.mret_i             (id_ex_mret)

);

// EX/MEM Register

ex_mem_reg u_ex_mem_reg (

.clk_i              (clk_200m),

.rst_n_i            (rst_n_internal),

.stall_i            (stall_mem),

.flush_i            (flush_mem),

.intr_flush_i       (intr_flush_mem),

.ex_alu_result_i    (ex_alu_result),

.ex_mem_addr_i      (ex_mem_addr),

.ex_mem_wdata_i     (ex_mem_wdata),

.ex_pc_plus4_i      (ex_pc_plus4),

.ex_rd_addr_i       (ex_rd_addr),

.ex_mem_we_i        (ex_mem_we),

.ex_mem_re_i        (ex_mem_re),

.ex_mem_width_i     (ex_mem_width),

.ex_wb_sel_i        (ex_wb_sel),

.ex_reg_we_i        (ex_reg_we),

.ex_csr_result_i    (ex_csr_result),

.mem_alu_result_o   (ex_mem_alu_result),

.mem_mem_addr_o     (ex_mem_mem_addr),

.mem_mem_wdata_o    (ex_mem_mem_wdata),

.mem_pc_plus4_o     (ex_mem_pc_plus4),

.mem_rd_addr_o      (ex_mem_rd_addr),

.mem_mem_we_o       (ex_mem_mem_we),

.mem_mem_re_o       (ex_mem_mem_re),

.mem_mem_width_o    (ex_mem_mem_width),

.mem_wb_sel_o       (ex_mem_wb_sel),

.mem_reg_we_o       (ex_mem_reg_we),

.mem_csr_result_o   (ex_mem_csr_result)

);

assign ex_mem_rd_addr_for_hazard = ex_mem_rd_addr;

assign ex_mem_reg_we_for_hazard = ex_mem_reg_we;

assign ex_mem_mem_re_for_hazard = ex_mem_mem_re;

assign forward_mem_data = mem_wb_mem_re ? wb_mem_rdata : wb_alu_result;

// MEM Top

mem_top_fpga u_mem_top (

.clk_i             (clk_200m),

.rst_n_i           (rst_n_internal),

.alu_result_i      (ex_mem_alu_result),

.wdata_i           (ex_mem_mem_wdata),

.mem_we_i          (ex_mem_mem_we),

.mem_re_i          (ex_mem_mem_re),

.mem_width_i       (ex_mem_mem_width),

.pc_plus4_i        (ex_mem_pc_plus4),

.reg_we_i          (ex_mem_reg_we),

.wb_sel_i          (ex_mem_wb_sel),

.rd_addr_i         (ex_mem_rd_addr),

.pc_plus4_o        (mem_pc_plus4),

.reg_we_o          (mem_reg_we),

.wb_sel_o          (mem_wb_sel),

.rd_addr_o         (mem_rd_addr),

.bus_re_o          (mem_bus_re),

.bus_we_o          (mem_bus_we),

.bus_addr_o        (mem_bus_addr),

.bus_wdata_o       (mem_bus_wdata),

.bus_width_o       (mem_bus_width),

.bus_rdata_i       (mem_bus_rdata),

.bus_ready_i       (mem_bus_ready),

.mem_exception_o   ()

);

// MEM/WB Register

mem_wb_reg u_mem_wb_reg (

.clk_i            (clk_200m),

.rst_n_i          (rst_n_internal),

.stall_i          (stall_wb),

.flush_i          (flush_wb),

.intr_flush_i     (intr_flush_wb),

.mem_alu_result_i (ex_mem_alu_result),

.mem_mem_rdata_i  (mem_bus_rdata),

.mem_pc_plus4_i   (mem_pc_plus4),

.mem_rd_addr_i    (mem_rd_addr),

.mem_wb_sel_i     (mem_wb_sel),

.mem_reg_we_i     (mem_reg_we),

.mem_csr_result_i (ex_mem_csr_result),

.mem_mem_re_i     (ex_mem_mem_re),

.wb_mem_re_o      (mem_wb_mem_re),

.wb_alu_result_o  (wb_alu_result),

.wb_mem_rdata_o   (wb_mem_rdata),

.wb_pc_plus4_o    (wb_pc_plus4),

.wb_rd_addr_o     (wb_rd_addr),

.wb_wb_sel_o      (wb_wb_sel),

.wb_reg_we_o      (wb_reg_we),

.wb_csr_result_o  (wb_csr_result)

);

// WB Top

wb_top u_wb_top (

.clk_i          (clk_200m),

.rst_n_i        (rst_n_internal),

.alu_result_i   (wb_alu_result),

.mem_rdata_i    (wb_mem_rdata),

.pc_plus4_i     (wb_pc_plus4),

.csr_result_i   (wb_csr_result),

.wb_sel_i       (wb_wb_sel),

.reg_we_i       (wb_reg_we),

.rd_addr_i      (wb_rd_addr),

.wb_data_o      (wb_data),

.reg_we_o       (wb_reg_we_out),

.rd_addr_o      (wb_rd_addr_out)

);

// Bus Arbiter

bus_arbiter u_bus_arbiter (

.clk_i          (clk_200m),

.rst_n_i        (rst_n_internal),

.mem_re_i       (mem_bus_re),

.mem_we_i       (mem_bus_we),

.mem_addr_i     (mem_bus_addr),

.mem_wdata_i    (mem_bus_wdata),

.mem_width_i    (mem_bus_width),

.mem_rdata_o    (mem_bus_rdata),

.mem_ready_o    (mem_bus_ready),

.ram_re_o       (bus_ram_re),

.ram_we_o       (bus_ram_we),

.ram_addr_o     (bus_ram_addr),

.ram_wdata_o    (bus_ram_wdata),

.ram_width_o    (bus_ram_width),

.ram_rdata_i    (bus_ram_rdata),

.ram_ready_i    (bus_ram_ready),

.uart_we_o      (bus_uart_we),

.uart_re_o      (bus_uart_re),

.uart_addr_o    (bus_uart_addr),

.uart_wdata_o   (bus_uart_wdata),

.uart_rdata_i   (bus_uart_rdata),

.gpio_we_o      (bus_gpio_we),

.gpio_re_o      (bus_gpio_re),

.gpio_addr_o    (bus_gpio_addr),

.gpio_wdata_o   (bus_gpio_wdata),

.gpio_rdata_i   (bus_gpio_rdata),

.timer_we_o     (bus_timer_we),

.timer_re_o     (bus_timer_re),

.timer_addr_o   (bus_timer_addr),

.timer_wdata_o  (bus_timer_wdata),

.timer_rdata_i  (bus_timer_rdata),

.spi_we_o       (bus_spi_we),

.spi_re_o       (bus_spi_re),

.spi_addr_o     (bus_spi_addr),

.spi_wdata_o    (bus_spi_wdata),

.spi_rdata_i    (bus_spi_rdata),

.i2c_we_o       (bus_i2c_we),

.i2c_re_o       (bus_i2c_re),

.i2c_addr_o     (bus_i2c_addr),

.i2c_wdata_o    (bus_i2c_wdata),

.i2c_rdata_i    (bus_i2c_rdata)

);

// UART Controller

uart_ctrl #(

.CLK_FREQ(200_000_000),

.BAUD_RATE(115200)

) u_uart_ctrl (

.clk_i          (clk_200m),

.rst_n_i        (rst_n_internal),

.we_i           (bus_uart_we),

.addr_i         (bus_uart_addr),

.wdata_i        (bus_uart_wdata),

.rdata_o        (bus_uart_rdata),

.tx_pin_o       (uart_tx_o),

.tx_data_o      (),

.tx_valid_o     (),

.debug_state_o  (),

.debug_baud_cnt_o(),

.debug_bit_cnt_o(),

.debug_shift_reg_o(),

.debug_fifo_data0_o(),

.debug_fifo_data1_o(),

.debug_fifo_data2_o(),

.debug_fifo_data3_o(),

.debug_fifo_data4_o(),

.debug_fifo_data5_o(),

.debug_fifo_data6_o(),

.debug_fifo_data7_o(),

.debug_fifo_data8_o(),

.debug_fifo_data9_o(),

.debug_fifo_data10_o(),

.debug_fifo_data11_o(),

.debug_fifo_data12_o(),

.debug_fifo_data13_o(),

.debug_fifo_data14_o(),

.debug_fifo_data15_o(),

.debug_wr_ptr_o  (),

.debug_rd_ptr_o  (),

.debug_fifo_count_o(),

.debug_fifo_full_o(),

.debug_fifo_empty_o(),

.debug_fifo_we_o (),

.debug_fifo_re_o (),

.debug_fifo_we_reg_o(),

.direct_transfer_o(),

.data_to_send_o  (),

.tx_ready_o      ()

);

// GPIO

gpio u_gpio (

.clk_i        (clk_200m),

.rst_n_i      (rst_n_internal),

.we_i         (bus_gpio_we),

.re_i         (bus_gpio_re),

.addr_i       (bus_gpio_addr),

.wdata_i      (bus_gpio_wdata),

.rdata_o      (bus_gpio_rdata),

.gpio_in_i    (gpio_in_reg),

.gpio_out_o   (gpio_out_reg),

.gpio_oe_o    (gpio_oe_reg),

.interrupt_o  (gpio_interrupt),

.debug_gpio_out(),

.debug_gpio_oe (),

.debug_gpio_in (),

.debug_gpio_if ()

);

// Timer

timer u_timer (

.clk_i        (clk_200m),

.rst_n_i      (rst_n_internal),

.we_i         (bus_timer_we),

.re_i         (bus_timer_re),

.addr_i       (bus_timer_addr),

.wdata_i      (bus_timer_wdata),

.rdata_o      (bus_timer_rdata),

.interrupt_o  (timer_interrupt),

.debug_load_value(),

.debug_counter(),

.debug_enable(),

.debug_irq_flag()

);

// SPI Master

spi_master u_spi_master (

.clk_i          (clk_200m),

.rst_n_i        (rst_n_internal),

.we_i           (bus_spi_we),

.re_i           (bus_spi_re),

.addr_i         (bus_spi_addr),

.wdata_i        (bus_spi_wdata),

.rdata_o        (bus_spi_rdata),

.sclk_o         (spi_sclk_o),

.mosi_o         (spi_mosi_o),

.miso_i         (spi_miso_i),

.cs_o           (spi_cs_o),

.interrupt_o    (spi_interrupt),

.debug_state_o  (),

.debug_tx_data_o(),

.debug_rx_data_o()

);

// I2C Master

i2c_master u_i2c_master (

.clk_i          (clk_200m),

.rst_n_i        (rst_n_internal),

.we_i           (bus_i2c_we),

.re_i           (bus_i2c_re),

.addr_i         (bus_i2c_addr),

.wdata_i        (bus_i2c_wdata),

.rdata_o        (bus_i2c_rdata),

.sda_io         (i2c_sda_io),

.scl_io         (i2c_scl_io),

.interrupt_o    (i2c_interrupt),

.debug_state_o  (),

.debug_tx_data_o(),

.debug_rx_data_o(),

.debug_ack_o    ()

);

// CSR Regfile

csr_regfile u_csr_regfile (

.clk_i              (clk_200m),

.rst_n_i            (rst_n_internal),

.csr_addr_i         (id_ex_csr_addr),

.csr_rdata_o        (csr_rdata),

.csr_inst_we_i      (csr_inst_we),

.csr_inst_waddr_i   (csr_inst_waddr),

.csr_inst_wdata_i   (csr_inst_wdata),

.csr_mepc_we_i      (pipe_csr_mepc_we),

.csr_mepc_data_i    (pipe_csr_mepc_data),

.csr_mcause_we_i    (pipe_csr_mcause_we),

.csr_mcause_data_i  (pipe_csr_mcause_data),

.csr_mstatus_we_i   (pipe_csr_mstatus_we),

.csr_mstatus_data_i (pipe_csr_mstatus_data),

.intr_software_i    (intr_software),

.intr_timer_i       (intr_timer),

.intr_external_i    (intr_external),

.mtvec_o            (mtvec),

.mepc_o             (mepc),

.mcause_o           (mcause),

.mie_o              (mie),

.mstatus_o          (mstatus),

.mip_o              (mip)

);

// CSR Instructions

csr_instructions u_csr_instructions (

.clk_i              (clk_200m),

.rst_n_i            (rst_n_internal),

.csr_inst_valid_i   (csr_inst_valid),

.csr_op_i           (csr_op),

.csr_addr_i         (csr_addr),

.rs1_addr_i         (csr_rs1_addr),

.rs1_data_i         (csr_rs1_data),

.imm_i              (csr_imm),

.csr_rdata_i        (csr_rdata),

.csr_we_o           (csr_inst_we),

.csr_waddr_o        (csr_inst_waddr),

.csr_wdata_o        (csr_inst_wdata),

.csr_result_o       (csr_inst_result)

);

assign csr_inst_valid = id_ex_csr_inst;

assign csr_op         = id_ex_csr_op;

assign csr_addr       = id_ex_csr_addr;

assign csr_rs1_addr   = id_ex_rs1_addr;

assign csr_rs1_data   = op1_selected;

assign csr_imm        = id_ex_imm;

assign csr_result     = csr_inst_result;

// Interrupt Controller

interrupt_controller u_interrupt_controller (

.clk_i              (clk_200m),

.rst_n_i            (rst_n_internal),

.intr_software_i    (intr_software),

.intr_timer_i       (intr_timer),

.intr_external_i    (intr_external),

.mie_i              (mie),

.mip_i              (mip),

.mstatus_i          (mstatus),

.mtvec_i            (mtvec),

.intr_pending_o     (intr_pending),

.intr_cause_o       (intr_cause),

.intr_handler_addr_o(intr_handler_addr),

.intr_spi_i         (spi_interrupt),

.intr_i2c_i         (i2c_interrupt)

);

// Interrupt Pipeline

interrupt_pipeline u_interrupt_pipeline (

.clk_i              (clk_200m),

.rst_n_i            (rst_n_internal),

.if_pc_i            (if_pc),

.id_valid_i         (|if_id_instr),

.id_pc_i            (if_id_pc),

.ex_valid_i         (|id_ex_opcode),

.ex_pc_i            (id_ex_pc),

.ex_branch_taken_i  (ex_branch_taken),

.ex_jump_taken_i    (ex_jump_taken),

.mem_valid_i        (ex_mem_mem_we || ex_mem_mem_re),

.mem_pc_i           (ex_mem_pc_plus4 - 4),

.mem_mem_re_i       (ex_mem_mem_re),

.mem_mem_we_i       (ex_mem_mem_we),

.wb_valid_i         (wb_reg_we_out),

.wb_rd_addr_i       (wb_rd_addr_out),

.wb_reg_we_i        (wb_reg_we_out),

.intr_pending_i     (intr_pending),

.intr_cause_i       (intr_cause),

.mstatus_i          (mstatus),

.csr_mepc_we_o      (pipe_csr_mepc_we),

.csr_mepc_data_o    (pipe_csr_mepc_data),

.csr_mcause_we_o    (pipe_csr_mcause_we),

.csr_mcause_data_o  (pipe_csr_mcause_data),

.csr_mstatus_we_o   (pipe_csr_mstatus_we),

.csr_mstatus_data_o (pipe_csr_mstatus_data),

.interrupt_taken_o  (interrupt_taken_pipe),

.interrupt_flush_o  (interrupt_flush_pipe),

.interrupt_pc_o     (interrupt_pc_pipe)

);

// ==========================================================================

// Step 5: LED调试指示器

// ==========================================================================

// 时钟分频计数器，用于LED闪烁

reg [27:0] led_counter;

always @(posedge clk_200m) begin

if (!rst_n_internal) begin

led_counter <= 28'h0;

end else begin

led_counter <= led_counter + 1;

end

end

// LED分配方案

assign led0_o = rst_n_internal;           // LED0: 复位完成指示

assign led1_o = led_counter[26];          // LED1: 系统时钟活动（慢闪）

assign led2_o = |if_instr[31:0];          // LED2: 指令有效指示

assign led3_o = bus_uart_we;              // LED3: UART写使能

assign led4_o = |if_pc[31:0];             // LED4: PC非零指示

assign led5_o = ex_branch_taken;          // LED5: 分支发生

assign led6_o = ex_jump_taken;            // LED6: 跳转发生

assign led7_o = 1'b1;                     // LED7: 系统运行指示（常亮）

endmodule